Uniram Hybrid brochure (product sheet) - LS

Header:

The flexibility of an online dripper. The performance of UniRam.

Techline™ CV and HCV Hybrid combines the installation flexibility of an online dripper with the superior performance and reliability of Techline™ CV and HCV inline technology. It is designed for demanding residential and commercial landscape applications, including trees, shrubs, hedges, planting beds and challenging or sloped terrain.

Benefits & Features

- Online Dripper Flexibility – Accessories can be easily connected directly to the dripper outlet, providing greater flexibility for different landscape applications.

- Clogging resistance - Large Filtration Area and large flow path cross section – Provides increased protection against clogging and reliable performance in challenging water conditions.

- Pre-Welded Outlet – Provides a secure, leak-free connection between the dripper and tubing.

- Pressure Compensating – Delivers consistent flow and uniform irrigation across varying pressure conditions and long runs.

- Prevents drainage (CV) - The CV (Check-valve) mechanism maintains dripline pressure by preventing drainage from emitters located in low areas, ensuring uniform water distribution. Eliminates the drainage and refill effect, improving irrigation efficiency in pulse irrigation applications.

- Techline CV Reliability – Combines the proven performance and durability of Techline CV inline dripper technology with the flexibility of an online solution.

Applications.

Trees

Shrubs

Hedges

Planting beds

Challenging or sloped terrain.

| CV |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flow rate* (gph) | Working pressure range | Water passages dimmensions (width-depth-length (in) | Filtration area (mm2) | Constant K | Exponent* X | Recommended filtration (micron)/(mesh) | Shut off pressure (psi |
| 0.26 | 14 - 58 psi | 0.033 x 0.029 x 1.57 | 130 | 0.26 | 0 | 130 / 120 | 2 |
| 0.42 |  | 0.043 x 0.030 x 1.57 | 130 | 0.42 | 0 | 200 / 80 | 2 |
| 0.61 |  | 0.050 x 0.037 x 1.57 | 130 | 0.61 | 0 | 200 / 80 | 2 |
| 0.92 |  | 0.063 x 0.042 x 1.57 | 150 | 0.92 | 0 | 200 / 80 | 2 |
| HCV |  |  |  |  |  |  |  |
| Flow rate* (gph) | Working pressure range | Water passages dimmensions (width-depth-length (in) | Filtration area (mm2) | Constant K | Exponent* X | Recommended filtration (micron)/(mesh) | Shut off pressure (psi |
| 0.33 | 21 - 58 psi | 0.028 x 0.026 x 1.57 | 110 | 0.33 | 0 | 130 / 120 | 3.6 |
| 0.53 |  | 0.033 x 0.029 x 1.57 | 130 | 0.53 | 0 | 130 / 120 | 3.6 |
| 0.77 |  | 0.043 x 0.030 x 1.57 | 130 | 0.77 | 0 | 200 / 80 | 3.6 |

| Dripline Technical data |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inside diameter, (in) | Wall Thickness, (in) | Outside diameter, (in) | Nominal diameter (mm) | Maximum working pressure, (psi) | Maximum flushing pressure, (psi) | Kd |
| 0.570 | 0.045 | 0.660 | 17 mm | 55 | 72 | 1.20 |
| Dripline Pachage data |  |  |  |  |  |  |
| Inside diameter, (in) | Wall Thickness, (in) | Outside diameter, (in) | Nominal diameter (mm) | Coil Length, (ft) | Average Coils Weight (lb) | Coils on a Pallet, (Units) |
| 0.570 | 0.045 | 0.660 | 17 mm | 100 | TBD | TBD |
| 0.570 | 0.045 | 0.660 | 17 mm | 250 | TBD | TBD |
| 0.570 | 0.045 | 0.660 | 17 mm | 500 | TBD | TBD |
| 0.570 | 0.045 | 0.660 | 17 mm | 1000 | TBD | TBD |